ORG 0000H MOV SP, #70H MOV PSW, #00H LED EQU P1.6 ; Define BUZZER pin as P1.5 LED25 EQU P0.0 ; for showing 25% cycle LED50 EQU P0.1 ; for showing 50% cycle LED75 EQU P0.2 ; for showing 75% cycle LED100 EQU P0.3 ; for showing 100% cycle ;------------------------------------------------------------- RS EQU P1.1 EN EQU P1.2 ;------------------------------------------------------------- INITIALIZATION: MOV P0, #00H MOV P1, #00H MOV P2, #00H MOV P3, #0FEH ;11111110 ; for keypad ;------------------------------------------------------------- ;-------------------------------------------------------------- LCD_INIT: MOV R2, #38H ;INITIALIZING LCD, 2 LINES, 5X7 MATRIX ACALL COMMAND ;CALLING COMMAND SUBROUTINE MOV R2, #0EH ;DISPLAY ON, CURSOR ON ACALL COMMAND MOV R2, #01H ;CLEARING SCREEN ACALL COMMAND MOV A,#06H ;INCREMENT CURSOR POSITION ACALL COMMAND MOV R2, #80H ;CURSOR AT START OF 1ST LINE ACALL COMMAND MOV A,#3CH ACALL COMMAND MOV DPTR,#MSG ;Display Starting message ACALL LCD_PRINT ;Subroutine for displaying in LCD MOV DPTR,#MSG2 ACALL LCD_PRINT MOV R2,#01H ACALL COMMAND MOV DPTR,#MSG3 ;Display Starting message ACALL LCD_PRINT ;Subroutine for displaying in LCD MOV DPTR,#MSG4 ACALL LCD_PRINT MOV R2,#01H ACALL COMMAND MOV DPTR,#MSG5 ;Display Starting message ACALL LCD_PRINT ;Subroutine for displaying in LCD MOV DPTR,#MSG6 ACALL LCD_PRINT MOV R2,#01H ;for removing all the prev messages and start new ones ACALL COMMAND MOV DPTR,#MSG7 ;Display Starting message ACALL LCD_PRINT ;Subroutine for displaying in LCD MOV DPTR,#MSG8 ACALL LCD_PRINT MOV R2,#01H ;for removing all the prev messages and start new ones ACALL COMMAND MOV DPTR,#MSG9 ;Display Starting message ACALL LCD_PRINT ;Subroutine for displaying in LCD MOV DPTR,#MSG10 ACALL LCD_PRINT MOV R2,#01H ACALL COMMAND MOV DPTR,#MSG11 ;Display Starting message ACALL LCD_PRINT ;Subroutine for displaying in LCD MOV DPTR,#MSG12 ACALL LCD_PRINT MOV R2, #01H ; for removing all the prev messages and start new ones ACALL COMMAND MOV DPTR,#START1 ;Display Starting message ACALL LCD_PRINT ;Subroutine for displaying in LCD MOV DPTR,#START2 ACALL LCD_PRINT MOV R2, #01H ACALL COMMAND MOV DPTR,#ACTIVE ACALL LCD_PRINT

Upto this part we are actually printing the initial messages.

MSG:    DB 'EEE-4706',0

MSG2:   DB 'A1-GROUP-02',0

MSG3:   DB 'AFIF',0

MSG4:   DB 'ID:141',0

MSG5:   DB 'TASNIA',0

MSG6:   DB 'ID:113',0

MSG7:   DB 'FABBIHA',0

MSG8:   DB 'ID:105',0

MSG9:   DB 'FARUK',0

MSG10:   DB 'ID:133',0

MSG11:   DB 'SHERIFFO',0

MSG12:   DB 'ID:153',0

Initial messages; repeating same block of code to print all messages.

;LCD display subroutine----------------------------------------- LCD_PRINT: LOOP: CLR A MOVC A, @A+DPTR MOV R2, A JZ EXIT ;stop writing when null char is detected ACALL DISPLAY ;main display part ACALL DELAY ;giving a short delay for LCD INC DPTR LJMP LOOP EXIT: MOV R2, #0C0H ;go to the 2nd line of LCD ACALL COMMAND RET DISPLAY: MOV P2, R2 ;subroutine for displaying in LCD SETB RS ;RW is grounded in Hardware SETB EN ACALL DELAY CLR EN RET ;--------------------------------------------------------------- COMMAND: MOV P2, R2 ;giving instructions to LCD CLR RS SETB EN ACALL DELAY CLR EN ACALL DELAY RET ;--------------------------------------------------------------- DELAY: MOV 52H, #100 ;50 ms delay (approx) D1: MOV 51H, #255 DJNZ 51H, $ DJNZ 52H, D1 RET

This part is directly taken from lab-03 on lcd to create different subroutines for printing command.

Part-02:

DECISION: LJMP MOTOR_ROTATION ;Start Motor part ;MOTOR PART------------------------------------------------------ MOTOR_ROTATION: SETB P1.0 SETB P1.3 ;Input to motor driver CLR P1.4 ;Clockwise direction

From this part we dive into the starting of motor. We set P1.0 (enable pin of driver) to start the driver and our default direction of rotation is clockwise so we set IN1 high and IN2 low.(opposite for anticlockwise)

STARTUP_MOTOR: MOV R2, #01H ACALL COMMAND MOV DPTR, #INT_MSG ;print 'ENTER RPM LEVEL' LCALL LCD_PRINT MOV R2, #01H ACALL COMMAND MOV DPTR, #LEVEL ;print 'LEVEL: 1 TO A' LCALL LCD_PRINT LCALL DELAY MOV DPTR, #STOP ;print 'PRESS 0 TO STOP' LCALL LCD_PRINT MOV DPTR, #ACLCL LCALL LCD_PRINT

This part of the code displays all the messages and options of controlling the operation of motor.

This includes-

INT_MSG: DB 'ENTER RPM LEVEL',0 ;INITIAL MESSAGE

LEVEL: DB 'LEVELS: 1 TO A',0

STOP: DB 'PRESS 0 TO STOP',0

ACLCL: DB 'ACL: *     CL: #',0

Part-03: Key board scanning

In this part we are not following the conventional method described in the lab sheet. In the lab sheet one port was used as input and another one was used as output. Here we are using the port 3 for both rows and columns of keypad. (P3.0-P3.3 for rows and P3.4-P3.7 for columns). At the initialization section of code we defined MOV P3,#0FEH it denotes that in the upper 4 bits(P3.4-3.7;columns) e have 1111 like the code in lab sheet and in the lower four bits(P3.0-P3.3;rows) we have 1110(to start checking the 1st row)

;SCANNING ALL ROWS ------------------------------------------ ML: JNB P3.0, CC1 JNB P3.1, CC2 JNB P3.2, CC3 JNB P3.3, CC4 SJMP ML

Like I said earlier we already sent 1110 to the row so we will check whether any key was pressed in the column to get the corresponding digit from the keypad. For this we check if there is a zero value in the first row if so then we for the checking of corresponding column values .

To mention the structure of our keypad is:

Last row we have *,0,=,D instead of(F,0,E,D)

;SCANNING COLUMN1 ---------------------------------------------- CC1: JNB P3.4, JMP_TO_1 JNB P3.5, JMP_TO_2 JNB P3.6, JMP_TO_3 JNB P3.7, JMP_TO_A SETB P3.0 CLR P3.1 SJMP ML

Therefore we check whether we are getting a zero value is C1 C2 C3 C4 or not and based on that we jump to four different levels referring the symbols as per our keypad design of 1st row.

This is for the 1st row(JNB P3.0 we go to scan column 1). Similarly for rest of the rows we scan rest of the columns labeled by CC2,CC3,CC4

CC2: JNB P3.4, JMP_TO_4 JNB P3.5, JMP_TO_5 JNB P3.6, JMP_TO_6 JNB P3.7, JMP_TO_B SETB P3.1 CLR P3.2 SJMP ML ;SCANNING COLUMN3 --------------------------------------------- CC3: JNB P3.4, JMP_TO_7 JNB P3.5, JMP_TO_8 JNB P3.6, JMP_TO_9 JNB P3.7, JMP_TO_C SETB P3.2 CLR P3.3 SJMP ML ;SCANNING COLUMN4 --------------------------------------------- CC4: JNB P3.4, JMP_TO_F JNB P3.5, JMP_TO_0 JNB P3.6, JMP_TO_E JNB P3.7, JMP_TO_D SETB P3.3 CLR P3.0 LJMP ML

After checking 1st column, if none of the keys in the column were pressed then we need to go to the second row and need to send 1101 to the ports. That’s why we do –

SETB P3.0

CLR P3.1

SJMP ML .Set the second row  and first one 1 to send 1101 . Same for all other column operations.

ASCII LOOK-UP TABLE FOR EACH ROW

KCODE0: DB '1','2','3','A'    ;ROW 0

KCODE1: DB '4','5','6','B'    ;ROW 1

KCODE2: DB '7','8','9','C'    ;ROW 2

KCODE3: DB '*','0','=','D'    ;ROW 3

Part-04:

JMP_TO_0: LJMP M_0 ;MOTOR OFF JMP_TO_1: LJMP M_1 ;LEVEL 1 JMP_TO_2: LJMP M_2 ;LEVEL 2 JMP_TO_3: LJMP M_3 ;LEVEL 3 JMP_TO_4: LJMP ML ;NOT NEEDED JMP_TO_5: LJMP ML ;continue scanning JMP_TO_6: LJMP ML JMP_TO_7: LJMP ML JMP_TO_8: LJMP ML JMP_TO_9: LJMP ML JMP_TO_A: LJMP M_A ;LEVEL 4 JMP_TO_B: LJMP ML JMP_TO_C: LJMP ML JMP_TO_D: LJMP ML JMP_TO_E: LJMP M_E ;CLOCKWISE JMP_TO_F: LJMP M_F ;ANTI-CLOCKWISE

As mentioned earlier to take the input from the user to control the operation of motor we are using keypad. For that after getting an input from the keypad we jump to corresponding mode of operation we want to do. Here one thing to remember is that we are using only 1st and last row in our entire operation. Any input from 2nd and 3rd row will not affect the operation. That is why we have declared labels with the corresponding inputs from the 1st (1,2,3,A)and last rows (*,0,=,D) and for the rest of the rows we have used LJMP ML to go back to the initial stage of scanning keypad cause we donot want any input other than 1st and last row

Now we will define different mode of operation of the motor

M_0: JNB P3.5,M_0 ;wait for key release MOV R2, #01H ACALL COMMAND ;clear LCD screen MOV DPTR, #OFF_MSG ;display 'Motor off' ACALL LCD_PRINT LJMP MOTOR_0 ;MOTOR OFF STATE CLR LED ; Turn off LED when '0' is pressed

This label is used whenever we press 0 in the keyboard it will display the message :

OFF_MSG: DB 'MOTOR OFF',0

And to show that it will send command to LCD. After that it will go to the corresponding MOTOR_0 level to execute the task of stopping the motor. Similarly for the rest of the 4 steps.

M_1: JNB P3.4,M_1 MOV R2, #01H ACALL COMMAND MOV DPTR, #LVL1 ACALL LCD_PRINT MOV DPTR, #STOP ACALL LCD_PRINT LJMP MOTOR_1 ;RPM LEVEL 1 M_2: JNB P3.5,M_2 MOV R2, #01H ACALL COMMAND MOV DPTR, #LVL2 ACALL LCD_PRINT MOV DPTR, #STOP ACALL LCD_PRINT LJMP MOTOR_2 ;RPM LEVEL 2 M_3: JNB P3.6,M_3 MOV R2, #01H ACALL COMMAND MOV DPTR, #LVL3 ACALL LCD_PRINT MOV DPTR, #STOP ACALL LCD_PRINT LJMP MOTOR_3 ;RPM LEVEL 3 M_A: JNB P3.7,M_A MOV R2, #01H ACALL COMMAND MOV DPTR, #LVL4 ACALL LCD_PRINT MOV DPTR, #STOP ACALL LCD_PRINT LJMP MOTOR_4 ;RPM LEVEL 4 M_F: JNB P3.4,M_F MOV R2, #01H ACALL COMMAND MOV DPTR, #ANTICLK ;Display 'Anticlockwise' ACALL LCD_PRINT MOV DPTR, #CLM ACALL LCD_PRINT SETB P1.0 SETB P1.4 ;anticlockwise rotation CLR P1.3 LJMP ML ;scan for new input M_E: JNB P3.6,M_E MOV R2, #01H ACALL COMMAND MOV DPTR, #CLKWISE ACALL LCD_PRINT MOV DPTR, #CLM ACALL LCD_PRINT SETB P1.0 SETB P1.3 ;clockwise rotation (In1=1,In2=0) CLR P1.4 LJMP ML

LVL1: DB 'L1:75 RPM,25%PW',0

LVL2: DB 'L2:150 RPM,50%PW ',0

LVL3: DB 'L3:225 RPM,75%PW ',0

LVL4: DB 'L4:300 RPM,100PW',0

ANTICLK: DB 'ANTI-CLOCKWISE',0

CLM: DB 'CLK    : PRESS #',0

CLKWISE: DB 'CLOCKWISE',0

ACLM: DB 'ANTICLK: PRESS *',0

And in each level we are always printing the message: how to stop the motor. That is why it is always incorporated in each level (MOV DPTR, #STOP

ACALL LCD_PRINT)

Part-05:

MOTOR_0: CLR P1.0 ;MOTOR OFF CLR P1.3 CLR P1.4 JNB P3.4,LL ;JNB IS SHORT JUMP, JNB P3.5,LL ;SO LL IS USED AS AN INTERMEDIATE STAGE JNB P3.6,LL JNB P3.7,LL SETB P3.0 CLR P3.3 ;CHECK ROW 4 (* 0 # D) JNB P3.4,LC ;'*' PRESSED JNB P3.5,LC ;'0' PRESSED JNB P3.6,LC ;'#' PRESSED SETB P3.3 CLR LED ; Turn off LED when '0' is pressed CLR LED25 CLR LED50 CLR LED75 CLR LED100 CLR P3.0 ;AGAIN CHECK ROW 1(1 2 3 A) SJMP MOTOR_0

In this part to halt the motor we need to set 0 to IN1 and IN2(CLR P1.3 and P1.4). Then we check if any key is pressed from row 1 or last row(motor will again start). To do so we at first check if any column is pressed then go to the start of the keypad to check which data is pressed and do the corresponding operation. To check whether there is any input from the last row we set P3.0 to 1 and P3.3 to 0 and check whether *,0,# which one is pressed. Then it will jump to LC label.

LC: MOV P3, #0FEH ;INITIAL CONDITION

LJMP ML ;SCAN KEYPAD AGAIN

;---------------------------------------------------------------

LL: LJMP ML ;JUMP TO L1

;---------------------------------------------------------------

It initializes the port and jump to ML from where we start scanning the keypad to check after pressing 0 whether we want to start our motor or not. If after stopping the motor we press any keys from the 1st row (1,2,3,A) or last row (*,#) motor will again start.

MOTOR_1: ;25% DUTY CYCLE SETB LED25 CLR LED50 CLR LED75 CLR LED100 CLR LED; buzzer SETB P1.0 SETB P1.3 CLR P1.4 ACALL DELAYM CLR P1.3 CLR P1.4 ACALL DELAYM ACALL DELAYM ACALL DELAYM ACALL DELAYM JNB P3.4,LL ;CHECK FOR NEW INPUT JNB P3.5,LL ;KEY 1,2,3,A FOR LEVEL 1-4 JNB P3.6,LL JNB P3.7,LL SETB P3.0 CLR P3.3 JNB P3.4,ACL1_1 ;'*' FOR ANTI-CLOCKWISE JNB P3.5,LC ;GO OUT WHEN '0' IS PRESSED JNB P3.6,CL1_1 ;# FOR CLOCKWISE SETB P3.3 CLR P3.0 ;JNB P1.0, LIM1 LJMP MOTOR_1

There are 4 different labels of operation. Level-1,(25%PW),2-50%,3-75% and 4=100%. Motor_1 refers to the 25% PW level. So whenever we are operating at 25% PW level corresponding led25 will be turned on and other leds will be turned off. Default rotation is clockwise so we set IN1 (P1.3) to high value and IN2 (P1.4) to low value then call a delay. Then clear IN1 and IN2 and call delay 4 times so (1/4*100=25%). Then we check for new input from the first row to shift to another rpm level. If so we jump to LL and start scanning the keypad again as stated earlier. Now if we want to rotate the motor anticlockwise then we need to scan for any input from the last row. If 0 is pressed then we will go out of the loop and jump to Motor_0 to halt the operation of the motor. If any key other than 0 is pressed then we will go to the corresponding ACL_1 or CL_1 level. If no key is pressed it will remain in MOTOR_1 state.

ACL1_1:LJMP ACL1 CL1_1: LJMP CL1

An intermediate stage from which we will jump to ACL1 and CL1.

CL1:ACALL CL_MAIN ;FOR CLOCKWISE ROTATION LJMP MOTOR_1 ;CONTINUE SAME RPM LEVEL ACL1:ACALL ACL_MAIN LJMP MOTOR_1_ACL ;-----------------

Based on my input (whether want clockwise or anticlockwise) after coming to this label it will direct my operation. If we press clockwise(= from keypad) then it will just simply send the command to MOTOR_1 since default direction of rotation was clockwise. If * is pressed then it will go to ACL_MAIN.

ACL_MAIN: MOV R2, #01H ACALL COMMAND MOV DPTR, #ANTICLK ACALL LCD_PRINT MOV DPTR, #CLM ACALL LCD_PRINT RET

Here it will just print the message on the LCD and return back to ACL_1 main.

ANTICLK: DB 'ANTI-CLOCKWISE',0

CLM: DB 'CLK    : PRESS #',0

Similar for clockwise if = was pressed.

CL_MAIN: MOV R2, #01H ACALL COMMAND MOV DPTR, #CLKWISE ACALL LCD_PRINT MOV DPTR, #ACLM ACALL LCD_PRINT RET

CLKWISE: DB 'CLOCKWISE',0

ACLM: DB 'ANTICLK: PRESS *',0

After coming back to ACL_1 main it will now jump to MOTOR_1_ACL where same 25% operation of motor is defined for anticlockwise direction.

MOTOR_1_ACL: ;25% DUTY CYCLE SETB LED25 CLR LED50 CLR LED75 CLR LED100 CLR LED SETB P1.0 SETB P1.4 CLR P1.3 ACALL DELAYM CLR P1.4 CLR P1.3 ACALL DELAYM ACALL DELAYM ACALL DELAYM ACALL DELAYM JNB P3.4,LL_2 ;CHECK FOR NEW INPUT JNB P3.5,LL_2 ;KEY 1,2,3,A FOR LEVEL 1-4 JNB P3.6,LL_2 JNB P3.7,LL_2 SETB P3.0 CLR P3.3 JNB P3.4,ACL1 ;'*' FOR ANTI-CLOCKWISE JNB P3.5,LC1_2 ;GO OUT WHEN '0' IS PRESSED JNB P3.6,CL1 ;# FOR CLOCKWISE SETB P3.3 CLR P3.0 SJMP MOTOR_1_ACL

We start in the same way turn on the led25 for 25% PW operation and turn off the all other leds. Then set enable and set IN2 to 1 and IN1 to zero for anticlockwise rotation. Call delays (1:4) times for 25% cycle and check whether we want to stay in this loop or go for any other rpm level. If we want to check direction back to clockwise then it will jump to CL1 and then go back to MOTOR_1 for clockwise rotation. If no key is pressed then it will stay in this loop.

If this part is clear then we will do the same for 50%,75% and 100% PW

MOTOR_2: ;50% DUTY CYCLE CLR LED25 CLR LED75 CLR LED100 CLR LED SETB LED50 SETB P1.0 SETB P1.3 CLR P1.4 ACALL DELAYM ACALL DELAYM CLR P1.3 CLR P1.4 ACALL DELAYM ACALL DELAYM JNB P3.4,LL_1 JNB P3.5,LL_1 JNB P3.6,LL_1 JNB P3.7,LL_1 SETB P3.0 CLR P3.3 JNB P3.4,ACL2_1 ;'*' pressed JNB P3.5,LC1_1 JNB P3.6,CL2_1 SETB P3.3 CLR P3.0 ;JNB P1.0, LIM2 SJMP MOTOR_2

ACL2_1: LJMP ACL2 CL2_1: LJMP CL2 CL2: ACALL CL_MAIN LJMP MOTOR_2 ACL2: ACALL ACL_MAIN ;rotation anticlockwise LJMP MOTOR_2_ACL ;continue with the same RPM ACL_MAIN: MOV R2, #01H ACALL COMMAND MOV DPTR, #ANTICLK ACALL LCD_PRINT MOV DPTR, #CLM ACALL LCD_PRINT RET MOTOR_2_ACL: ;50% DUTY CYCLE CLR LED25 CLR LED75 CLR LED100 CLR LED SETB LED50 SETB P1.0 SETB P1.4 CLR P1.3 ACALL DELAYM ACALL DELAYM CLR P1.4 CLR P1.3 ACALL DELAYM ACALL DELAYM JNB P3.4,LL_12 JNB P3.5,LL_12 JNB P3.6,LL_12 JNB P3.7,LL_12 SETB P3.0 CLR P3.3 JNB P3.4,ACL2 ;'*' pressed JNB P3.5,LC1 JNB P3.6,CL2 SETB P3.3 CLR P3.0 ;JNB P1.0, LIM2A SJMP MOTOR_2_ACL

75% PW

MOTOR_3: ;75% DUTY CYCLE CLR LED CLR LED25 CLR LED50 CLR LED100 SETB LED75 SETB P1.0 SETB P1.3 CLR P1.4 ;SETB LED75 ACALL DELAYM ACALL DELAYM ACALL DELAYM CLR P1.3 CLR P1.4 ACALL DELAYM ACALL DELAYM ACALL DELAYM ACALL DELAYM JNB P3.4,LL1_12 JNB P3.5,LL1_12 JNB P3.6,LL1_12 JNB P3.7,LL1_12 SETB P3.0 CLR P3.3 JNB P3.4,ACL3_1 JNB P3.5,LC1 JNB P3.6,CL3_1 SETB P3.3 CLR P3.0 ;JNB P1.0, LIM3 SJMP MOTOR_3

ACL3_1:LJMP ACL3 CL3_1:LJMP CL3 CL3:ACALL CL_MAIN LJMP MOTOR_3 ACL3:ACALL ACL_MAIN LJMP MOTOR_3_ACL MOTOR_3_ACL: ;75% DUTY CYCLE CLR LED CLR LED25 CLR LED50 CLR LED100 SETB LED75 SETB P1.0 SETB P1.4 CLR P1.3 ACALL DELAYM ACALL DELAYM ACALL DELAYM CLR P1.4 CLR P1.3 ACALL DELAYM ACALL DELAYM ACALL DELAYM ACALL DELAYM JNB P3.4,LL1_1 JNB P3.5,LL1_1 JNB P3.6,LL1_1 JNB P3.7,LL1_1 SETB P3.0 CLR P3.3 JNB P3.4,ACL3 JNB P3.5,LC11_1 JNB P3.6,CL3 SETB P3.3 CLR P3.0 ;JNB P1.0, LIM3A LJMP MOTOR_3_ACL

100%

MOTOR_4: ;KEY 'A' CLR LED25 CLR LED50 CLR LED75 SETB LED100 SETB P1.0 ;100% DUTY CYCLE SETB P1.3 CLR P1.4 SETB LED ; Turn on LED when motor reaches 3000 RPM ACALL DELAYM ACALL DELAYM ACALL DELAYM ACALL DELAYM JNB P3.4,LL1_2 JNB P3.5,LL1_2 JNB P3.6,LL1_2 JNB P3.7,LL1_2 SETB P3.0 CLR P3.3 JNB P3.4,ACL4_1 JNB P3.5,LC111_1 JNB P3.6,CL4_1 SETB P3.3 CLR P3.0 ;JNB P1.0, LIM4 CLR P1.0 SJMP MOTOR_4

ACL4_1:LJMP ACL4 CL4_1: LJMP CL4 CL4:ACALL CL_MAIN LJMP MOTOR_4 ACL4:ACALL ACL_MAIN SJMP MOTOR_4_ACL OTOR_4_ACL: ;KEY 'A' CLR LED25 CLR LED50 CLR LED75 SETB LED100 SETB P1.0 ;100% DUTY CYCLE SETB P1.4 CLR P1.3 ACALL DELAYM ACALL DELAYM ACALL DELAYM ACALL DELAYM JNB P3.4,LL1 JNB P3.5,LL1 JNB P3.6,LL1 JNB P3.7,LL1 SETB P3.0 CLR P3.3 JNB P3.4,ACL4 JNB P3.5,LC111_2 JNB P3.6,CL4 SETB P3.3 CLR P3.0 ;JNB P1.0, LIM4A CLR P1.0 LJMP MOTOR_4_ACL

That is all.

For overload protection when the motor is operating at 25% PW meaning that load is excess and speed is very low. In this regard what we can do is whenever we are setting the rpm level to 25% PW after a few seconds of operation the enable pin of the driver can be set to 0 so that the motor is disconnected. So we need to change the part of code MOTOR_1 and MOTOR_1 ACL a bit to do this task.